Adding a Received Signal Strength Meter to a Packet Station

Dave Roberts, G8KBB

# Introduction

During the summer of 1993 Neville Pattinson, G0JVU, designed an add-on Analogue to Digital Converter ( ADC ) board for a TNC2 Packet switch [1]. The board carried a four channel ADC that was connected to the bus of the TNC's Z80 processor allowing fast measurement of four channels of analogue data. It was packaged as a small PCB that plugged into the 40 pin processor socket in the TNC and carried a socket for the processor plus the ADC circuitry. It was employed in the TheNet X-1J software to implement a receive signal deviation meter, the software being based on TheNet 1.01 from NORD<>LINK

The Deviation meter worked by reading the peak audio level arriving at the TNC from the receiver. A precision rectifier converted the AC signal into a DC representation which was sampled by the ADC at the end of each packet. A fast ADC was needed to ensure correct capture of short packets given the DC settling time of the analogue circuits.

This article describes an extension to the use of the ADC, namely the addition of a signal strength meter to the system. Together with the deviation meter circuitry, the unit allows a remote station's transmit deviation and carrier level to be measured on each and every packet. It therefore has application beyond simply setting up a packet station as it may be used to assess link performance automatically from a large number of stations at the same time, thereby creating the opportunity for new services. An example of the type of service that might benefit from this is the one run by Dirk, G1TLH, who produces regular beacon analysis bulletins in East Anglia. It also has application to network routing control.

The circuitry for the deviation meter was part of the original design of the board and the signal strength meter does not require any changes to the existing circuit apart from connection to one of the unused pads on the ADC board.

It would be a simple matter to adapt the KISS protocol to allow the unit to be used for PC based systems that do not use TheNet X-1 by returning the ADC readings as bytes in the KISS frame structure. This would however need to be supported by suitable changes to the host end software.

Constructional details are brief, as this project is not for the faint hearted. Given the sheer range of types of radio used for packet and the variation of signal levels in receivers, the circuit should be viewed as a design concept that will need to be adapted to the specific radio rather than a "build to spec" type design.

# Theory of Operation

The requirements for the signal strength meter are :

·	It should be simple to make

·	It should be reasonably accurate

·		It should be stable and repeatable in its readings

·	It should have a wide operating range

Recent receiver designs, notably those based on the Motorola MC3356 / 3362 series devices have a logarithmic Receive Signal Strength Indicator ( RSSI ) function that is surprisingly accurate. This has already been noted in previous designs such as the 'Simple Spectrum Analyser' by Roger Blackwell, G4PMK [2]. Checking back for the references for this article, it is interesting to note that Roger also used this device in his design, a testament to its versatility. Cellular radio IF devices would also work very well.

If a receiver already has a suitable meter output of this type, it may be used for this application, but if not then these devices make an excellent basis for a meter. In the prototype meter created for this project, the radio used was an old Storno VHF PMR receiver on two metres with no signal strength output. A surplus MC3356 chip was used to create a meter by connecting it to the 455 KHz IF of the receiver after the 10.7 MHz mixer.

Figure 1 shows the prototype circuit. A buffer circuit is used to reduce loading on the existing circuit, and the signal from the buffer is filtered to leave only the desired signal. The low pass filter was needed as there was a significant component of the 11.155 MHz second conversion oscillator signal in the mixer output but a ceramic bandpass would work just as well. The output of the filter is fed into the limiting IF amplifier of the FM receiver chip and the output is taken from the meter output. The input mixer, oscillator, discriminator, squelch and comparator are not needed in this application. Note that the MC3356 is a wideband FSK receiver chip hence the comparator rather than an audio amplifier. Table 1 shows the parts list.

Other devices in the family may be used instead of the MC3356 provided that they have a suitable meter output. The advantage of the 3356 was that it was firstly available in my junk box and secondly it operates at IF frequencies up to 50 MHz without the need for additional mixers allowing the circuit to be tried in a range of receivers [3].

The output of the chip is a DC voltage that corresponds to the input signal in a logarithmic manner and over a wide range. Table 2 shows the calibration data for the prototype, created by applying a signal at 144.625 MHz to the aerial input of the receiver through a good quality attenuator and measuring the DC output level of the chip. It is also shown graphically in figure 2.

This DC voltage is fed to the channel two input to the ADC, channel one being used for the deviation meter function. The software in the TNC ( assuming it is designed to do so ) will sample the voltage at the end of each valid frame. This is converted into an indication of the signal strength. The circuit should be set so that the maximum DC level into the ADC does not exceed the three volt limit set by Neville's design. This is done by adjusting the value of R3.

# Constructional Details

The circuit is so simple that the easiest way of tackling it is to 'dead bug' the components on a small piece of copper clad board. The input filter will need to be changed if a different IF frequency is used, or it may be replaced by a ceramic bandpass filter. Care should be taken to ensure that the device is properly decoupled ( keep the leads short ) as the device is capable of operation at up to 200 MHz.

The input signal should be taken from the earliest possible point in the circuit where the desired signal is present. Typically this will be immediately after the most selective bandpass filter, usually a 10.7 MHz crystal filter, but may also be taken from the second conversion mixer to 455 KHz as was done in the prototype. The reason for this was simple - the signal lead terminations were more accessible at 455 KHz in this particular receiver than the 10.7 MHz signal. It must be taken from a point prior to any limiter stage. Extra gain or attenuation may be needed given the exact receiver circuit design, but in most receivers the signal levels will be about right. Note that the input impedance of the receiver IF is 330 ohms.

The meter output must be connected to the ADC chip in the TNC. The easiest way to do this is to use the squelch input in the five core audio lead usually found in TNCs ( Pin 5 in TNC2 clones such as the BSX2 ) as this signal is often unused in packet systems. The corresponding changes need to be effected to the TNC to disconnect pin 5 from the internal squelch detect functions. The simplest way to do this for a BSX2 or similar is to lift one end of D5 [4].

The five volt regulator chip needs a DC voltage in excess of eight volts in order to derive a stable five volts for the MC3356 receiver chip. Strictly speaking,  it is unnecessary to provide such a stabilised supply provided the DC limits of the receiver chip are met, but in most radios such a clean stable supply may not be provided. Take the safe route therefore and give it its own regulator. The IF circuits of the MC3356 will operate at up to nine volts DC supply.

# Alignment and calibration

This is essential. Without calibration, the meter can be set to show a familiar 'S1 to S9' type display, but calibrated dBm are far more useful.

Figure 2 shows the calibration curve for the prototype. It can be seen that the device is surprisingly linear with a usable range of over 60 dB and a maximum deviation from linear of less than 1dB apart from at the extremes of its range. At the top end of the range, it suffers from overload and at the bottom end of the range it has a strange 'kink'. This 'kink' has hysteresis to it and appears at about 0.8 V DC output from the chip. It is a manifestation of the chip's squelch circuits that open at about that voltage as can be seen in figure 3 which shows the transition region in detail for an increasing and a decreasing signal level. It may be that other chips would not show this characteristic, but operationally it is of minor significance as it corresponds to an area of operations that is marginal for packet radio. It could also be eliminated by using a six volt range for the meter and eight volt DC supply voltage to the MC3356 then dividing the output voltage by two using a resistive divider to give the three volt maximum for the ADC as shown in figure 4.

To calibrate it, readings of the DC voltage were taken by applying a known signal level from a signal generator through a switched attenuator to the aerial input of the receiver. The signal level was raised in 2 dB increments from -120 dBm to -50 dBm ( or according to the receiver in question ). The DC voltage was read at each point. The measurements were repeated to check for errors. In calibrating the prototype, two attenuators were used, one of 10 dB per step and one of 1 dB per step. The calibration process revealed slight discrepancies in the step accuracy of the 10 dB per step attenuator. These need to be subtracted out from the calibration data in order to get a true picture.

The calibration curve is plotted as a graph, and a number of calculations made. A 'best fit' straight line is drawn through the more linear part of the curve as shown in figure 5, and a zero point found that corresponds to the noise floor of the system and the corresponding dBm input. The number of dB per volt can then be found from the slope of the straight line. Note that this is the zero point of the meter which may not be the same as the noise floor of the receiver if the signal levels are wrong.

In the TheNet X-1 application, these three data items are input as parameters to the software that calculates the 'S' meter values and the dBm display as described later.

# Receiver Circuits

If a signal strength indicator exists on a receiver there is no need to build the MC3356 circuitry, it is only necessary to ensure that its output is in the range 0 to 3V DC or can be suitably scaled to ensure that the ADC is not overloaded and that it possesses the necessary logarithmic characteristics. If not, then a suitable tap point must be identified and the S meter circuit redesigned to suit the signal levels and frequency. Attention must be paid to components of the signal other than at the desired frequency, for example 10.245 MHz second conversion oscillators. The tap point must be after the main receiver selectivity but before any limiting amplifiers within the FM receiver's own IF strip. If the tap point in the receiver can take a 300 ohm load directly the TR1 buffer may be omitted. Do not omit C10 however. Signal levels may need adjustment so that the noise floor of the receiver is roughly the same as that of the meter. R3 ( and R6 if used ) may need adjustment to set the output range to 3V full scale. In doing this, it is worth noting that the 3356 chip gives out a fixed current proportional to the signal level.

# Node Software

At the time of writing the only software that implements support for this meter is a variant of TheNet X-1J used for beta testing the next release. The next full release will probably be a joint development with the author of TheNet Plus, Bill Beech NJ7P but an interim release of X-1 will contain the S meter function. As stated earlier, it would be simple to modify the KISS protocol to attach the deviation and signal strength readings to KISS frames. A suitable approach would be to modify the data structures in a manner similar to Multidrop KISS to include two extra bytes of data after the channel or command bytes. End user software such as is included in most TNCs will be difficult to adapt without manufacturer support.

The software extensions to TheNet X-1J convert the readings to signal levels by using the calibration data derived from testing. The noise floor is entered as the zero point by converting it to an ADC reading, so for a reading ( say ) of 0.65 V with an ADC reference voltage ( Vref ) of 3V, the zero point is entered as 256 * 0.65 / 3, i.e. a value of 55. The 256 comes from the fact that the ADC is an 8 bit device, hence 256 steps. The corresponding dBm zero point is entered as a positive value; for the prototype shown this was 113 ( i.e. -113 dBm ). For a swing of V volts for a change of X dB ( i.e. the slope of the graph ), the 'dB multiplier' is set as follows :

µ §

In the prototype data shown this was 54 dB for a two volt swing, giving a multiplier value of 81.

The dB reading is obtained by the software by the following formula :

µ §

where input_value is the input count in the range 0 to 255.

Readings are shown in the node's heard list, a typical display being shown in figure 6. Figure 7 shows a similar 'S meter' type display. Note that the deviation meter in these tables is not correctly calibrated. It is left in as it illustrates the 'signature' effect of a station.

# CONCLUSIONS

Powerful meter capabilities can be added fairly easily to a packet system. It requires suitable software and some technical ability. The results however are quite fascinating and may be of use in propagation experiments or in network management of packet systems. For close ( line of sight ) paths, the readings are consistent to within 1 dB. It would be interesting to combine the output of the meter with routing table data in deciding dynamically which route to take for passing packet data. It will also be found that each station has a fairly unique signature of signal level and deviation. From this it is possible to identify where two callsigns originate from the same station for example a user downlink from a node and the node itself.

# REFERENCES

[1]	"A Deviation Meter for TheNet X-1J", Neville Pattinson, G0JVU, July 1993, Distributed by packet radio, BBS and Internet

[2]	"Simple Spectrum Analyser", G4PMK, Radcom November 1989.

[3]	Motorola Data Sheet, MC3356

[4]	G0BSX Mark 2 TNC Documentation, Issue 5, August 1990

Figure 1 - Circuit Diagram

circuit

Table 1 - Parts list for prototype

| Item | Quantity | Reference | Value |
| --- | --- | --- | --- |
| 1 | 3 | C1,C2,C3 | 10n |
| 2 | 6 | C4,C6,C7,C8,C10,C11 | 100n |
| 3 | 1 | C5 | 10u |
| 4 | 1 | C9 | 560p |
| 5 | 1 | IC1 | MC3356 |
| 6 | 1 | IC2 | 7805 |
| 8 | 2 | L1,L2 | 56u |
| 9 | 1 | R1 | 330R |
| 10 | 1 | R2 | 82R |
| 11 | 1 | R3 |  |
| 12 | 1 | R4 | 100K |
| 13 | 1 | R5 | 180R |
| 14 | 1 | TR1 | J310 |

Table 2 - Calibration data for Prototype

| dBm | V(DC) |
| --- | --- |
| -120 | 0.65 |
| -118 | 0.65 |
| -116 | 0.65 |
| -114 | 0.654 |
| -112 | 0.662 |
| -110 | 0.673 |
| -108 | 0.727 |
| -106 | 0.927 |
| -104 | 0.976 |
| -102 | 1.048 |
| -100 | 1.12 |
| -98 | 1.19 |
| -96 | 1.28 |
| -94 | 1.36 |
| -92 | 1.44 |
| -90 | 1.51 |
| -88 | 1.58 |
| -86 | 1.65 |
| -84 | 1.73 |
| -82 | 1.82 |
| -80 | 1.9 |
| -78 | 1.98 |
| -76 | 2.05 |
| -74 | 2.12 |
| -72 | 2.19 |
| -70 | 2.26 |
| -68 | 2.34 |
| -66 | 2.42 |
| -64 | 2.5 |
| -62 | 2.57 |
| -60 | 2.63 |
| -58 | 2.69 |
| -56 | 2.75 |
| -54 | 2.82 |
| -52 | 2.88 |
| -50 | 2.95 |
| -48 | 3.01 |
| -46 | 3.07 |
| -44 | 3.13 |
| -42 | 3.17 |
| -40 | 3.2 |

Figure 2

µ §

Figure 3 - Closeup of Hysteresis part of curve

µ §

Figure 4 - Circuit modified for 6V operation

Figure 5 - Reference points in calibration data

µ §

Figure 6

Node heard list showing dBm format

IPNET:G8KBB-5}

Callsign    Pkts   Port  Time      Dev.   dBm   Type

G8KBB-2     1129   1     0:0:0                  Node TCP/IP

FELIX       869    0     0:0:6     5.7    -79

G0JVU-2     4285   0     0:0:40    5.9    -78   Node TCP/IP

G7MNS       368    0     0:1:17    4.1    -89

G8STW-5     6227   0     0:4:54    5.0    -102  TCP/IP

G1YRE       61     0     0:5:27    6.2    -82

GB7MXM      326    0     0:7:6     5.8    -78

FB1ICL      1      0     0:13:40   6.9    -104

G0TMH-5     1      0     0:13:57   6.1    -107  TCP/IP

G0OEY-5     2288   0     0:14:10   6.1    -93   Node TCP/IP

G1DVU-5     1      0     0:18:39   7.6    -107  TCP/IP

G8HUE       90     0     0:21:50   5.5    -92

G7BKO       1      0     2:0:14    7.0    -96

G4ZEK-14    13     0     3:39:22   5.7    -79

G0NJA       29     0     4:8:54    6.6    -91

G7JVE-5     259    0     5:23:33   4.3    -105  TCP/IP

G8INE       5      0     8:11:28   6.3    -112

G4IZC-5     69     0     8:26:29   6.8    -112  TCP/IP

Figure 7

Node heard list showing S-meter format

IPNET:G8KBB-5}

Callsign    Pkts   Port  Time      Dev.   Sig.  Type

G8KBB-2     1136   1     0:0:0                  Node TCP/IP

FELIX       869    0     0:0:28    5.7    S8

G0JVU-2     4285   0     0:1:2     5.9    S8    Node TCP/IP

G7MNS       368    0     0:1:39    4.1    S6

G8STW-5     6227   0     0:5:16    5.0    S2    TCP/IP

G1YRE       61     0     0:5:49    6.2    S7

GB7MXM      326    0     0:7:28    5.8    S8

FB1ICL      1      0     0:14:2    6.9    S2

G0TMH-5     1      0     0:14:19   6.1    S1    TCP/IP

G0OEY-5     2288   0     0:14:32   6.1    S5    Node TCP/IP

G1DVU-5     1      0     0:19:1    7.6    S1    TCP/IP

G8HUE       90     0     0:22:12   5.5    S5

G7BKO       1      0     2:0:36    7.0    S4

G4ZEK-14    13     0     3:39:44   5.7    S8

G0NJA       29     0     4:9:16    6.6    S5

G7JVE-5     259    0     5:23:55   4.3    S2    TCP/IP

G8INE       5      0     8:11:50   6.3    S0

G4IZC-5     69     0     8:26:51   6.8    S0    TCP/IP