Build Notes

- make sure that the variables used in foo.c at fixed locations match those in periph.c as foo.c cannot be allowed to allocate memory variables and must pick up parameters from periph.c

- build both projects. You will get errors from foo.c as it has no startup code

- combine the hex files as follows:
Open fw.hex and put the cursor before the last line
Open foo.hex and select all bar the last line
Paste foo.hex into fw.hex
Save fw.hex

Now do the same for fw500 creating fw500.hex. This is the 500mA version

Note PIDs used as follows

1004 – 100mA N2PK VNA before renumeration (programmed into eeprom)

1005 – as above after renumeration

1008 – 500mA version before renumeration

1009 – as above after renumeration